Yannick Kengne Tatcha

CPE301 – SPRING 2018

# Design Assignment 3

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| 1. | INITIAL CODE OF TASK 1/A |  |  |
| 2. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 2/B |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 3/C |  |  |
| 4. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 4/D |  |  |
| 5. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 5/E |  |  |
| 6. | SCHEMATICS |  |  |
| 7. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| 8. | SCREENSHOT OF EACH DEMO |  |  |
| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| 10. | GOOGLECODE LINK OF THE DA |  |  |

| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| --- | --- | --- | --- |

- Atmega328P

- LM34

- Pololu USB AVR programmer for serial communication

- AVR Studio 7

| 1. | INITIAL CODE OF TASK 1/A |  |  |
| --- | --- | --- | --- |

Task 1:  Write a C AVR program that will monitor the LM34/35 connected to an Analog pin to display the temperature in F on the serial terminal every 1 sec. Use a timer with interrupt for the 1 sec delay. Use a FTDI chip for serial to USB conversion.

Flow chart:

#define     F_CPU       8000000UL

#include    <stdio.h>

#include    <avr/io.h>

#include    <util/delay.h>

#include    <avr/interrupt.h>

#define BAUDRATE	9600    // set baudrate

#define ASYNCH_NORM_PRESCALER (F_CPU/16/BAUDRATE - 1) // prescaler value

int USART0_sendChar(char, FILE*);   // Send character on USART0

void USART0_init (void);            // Initialize USART0

void ADC0_init();                   // to initiate ADC

void TIMER1_init();                 // initiate timer1

// reset stream pointer

// http://www.gnu.org/savannah-checkouts/non-gnu/avr-libc/user-manual/group__avr__stdio.html

FILE USART0_stream = FDEV_SETUP_STREAM(USART0_sendChar, NULL, _FDEV_SETUP_WRITE);

int main(void)

{

stdout = &USART0_stream;   // change standard output to point to a USART stream

USART0_init();      // Initiate USART0

ADC0_init();        // Initiate ADC0

TIMER1_init();      // Initiate TIMER1

sei();              // enable global interrupts

printf("\n");       // Go to the next line in case current line printed junk.

while(1);           // infinite loop. Wait for interrupt.

return 0;

}

ISR(TIMER1_COMPA_vect)		// interrupt routine for Output Compare Match A.

// ISR that triggers at TIMER1_COMPT_vect

{

uint16_t ADC_value = ADC;	// read ADC conversion

float temperature_F;

c

temperature_F = (ADC_value / 1024.0) * 5000;

// 10 mV per degree Fahrenheit

temperature_F = temperature_F / 10; // Convert to Fahrenheit

printf("Temp(F) LM34: %3.2f\r", temperature_F);

}

void TIMER1_init()

// TIMER1_init initiates TIMER1 for 1 second delays to trigger an interrupt at 1 Hz

{

// WGM = 0100

TCNT1  = 0;

OCR1A  = 2*F_CPU/2/256-1;

TIMSK1 = (1<<OCIE1A);

TCCR1A = 0x00;

TCCR1B = (1<<WGM12) | (1<<CS12) | (0<<CS11) | (0<<CS10);

}

void ADC0_init()

// ADC0_init will initiate ADC acquisition at ADC0

{

DDRC &= ~(0<<DDC0);         // Clear bit 0 of DDRC

ADMUX = 0; // use ADC0

ADMUX |= (1 << REFS0);      // use AVcc as the reference

ADCSRA |= (1 << ADPS2) | (1 << ADPS1)/* | (1 << ADPS0)*/; // 128 prescale for 16Mhz

ADCSRA |= (1 << ADATE);     // Set ADC Auto Trigger Enable

ADCSRB = 0;                 // 0 for free running mode

ADCSRA |= (1 << ADEN);      // Enable the ADC

ADCSRA |= (1 << ADSC);      // Start the ADC conversion

}

int USART0_sendChar(char data, FILE *stream)

/*

* Procedure to send a single character over USART0. If character is linefeed, reset

* line.

* Assumes ASCII code.

*/

{

if(data == '\n') // If linefeed, also print a return.

{

while(! (UCSR0A & (1<<UDRE0)) );// Wait for data register to be available.

UDR0 = '\r';    // send return char to data register.

}

while(! (UCSR0A & (1<<UDRE0)) );    // Wait for data register to be available.

UDR0 = data;        // send char to UART.

return 0;

}

void USART0_init (void)

/*

* Procedure to initialize USART0 asynchronous with enabled RX/TX, 8 bit data,

* no parity, and 1 stop bit.

*/

{

UCSR0B = (1<<TXEN0)  | (1<<RXEN0);  // enable transmit/receive

UCSR0C = (1<<UCSZ01) | (1<<UCSZ00); // asynchronous, 8N1

UBRR0L = ASYNCH_NORM_PRESCALER;     // To set 9600 baud rate with 8MHz clock

}

| 6. | SCHEMATICS |  |  |
| --- | --- | --- | --- |

| 7. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| --- | --- | --- | --- |

TASK 1:

At a frequency of 1Hz, the current captured temperature was sent over UART and printed on the serial terminal. By using

printf("Temp(F) LM34: %3.2f\r", temperature_F);

The information could be printed on a single line since the return character ‘\r’ made the next incoming text print over the old.

Task2

| 8. | SCREENSHOT OF EACH DEMO |  |  |
| --- | --- | --- | --- |

Task1

| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| --- | --- | --- | --- |
| 10. | GITHUB REPOSITORY LINK OF THE DA |  |  |
| https://github.com/Vasty1995/CPE301 |  |  |  |

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Yannick Kengne Tatcha