Yannick Kengne Tatcha

CPE301 – SPRING 2018

# Midterm

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 1. | COMPONENTS LIST |  |  |
| 2. | TASK 1 & 2 COMPLETE CODE W/ COMMENTS AND SCREENSHOTS |  |  |
| 3. | AT FIRMWARE DOWNLOAD AND VERSION CONFIRMATION |  |  |
| 4. | THINGSPEAK ACCOUNT AND CHANNEL KEY |  |  |
| 5. | FLASH AND TRANSMISSION |  |  |
| 6. | THINGSPEAK GRAPH |  |  |

- COMPONENTS LIST

Needed:

FTDI232R chip

ATMEGA328P Microcontroller

LM34 Sensor

ESP8266-01

- TASK 1 & 2 COMPLETE CODE W/ COMMENTS AND SCREENSHOTS

/*

* midterm1.c

*

* Created: 4/11/2018 11:09:31 AM

* Author : YKengne

*/

#include <stdlib.h>

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#include <stdio.h>

#include <stdint.h>

#define F_CPU 8000000UL

#define FOSC 16000000			//Frequency

#define BAUD 9600				//Baud Rate

#define MYUBRR FOSC/16/BAUD-1	//Automatic BAUD rate calculation

volatile uint8_t adcValue;

volatile uint8_t fifteenPlus = 120;	//

/*Function Declarations*/

void USART_Init();

void ADC_Init();

void outputChr(unsigned char c);

void outputStr(char *c);

void readTemp();

void AT_Tx(char *t);

ISR(ADC_vect)

{

ADCSRA	|= (1 << ADIF);	//Reset flag

adcValue = ADCH;		//MSB 8-bits of ADC form left shift of ADLAR

}

ISR(TIMER1_OVF_vect)

{

TIFR1 |= (1 << TOV1);	//Clr Flag

fifteenPlus++;

}

int main(void)

{

TCCR1B	|= (1 << CS12) | (1 << CS10);	//Set prescale 1024

TIMSK1 |= (1 << TOIE1);					//enable OVF interrupt

unsigned int oneFive = 0;

ADC_Init();			//initialize ADC

USART_Init();		//UART initialization

while (1)

{

if(fifteenPlus>=120)

{

readTemp();

fifteenPlus = 0;

}

}

}

void USART_Init()

{

UBRR0H = (MYUBRR>>8);	//Shift MSB "top" of UBRR0H	0100 0100 >> 8 -> UBRR0H 0000 0000

UBRR0L = MYUBRR;		//UBRR0L 0100 0100

UCSR0B |= (1 << RXEN0) | (1 << TXEN0);	 //Enable Rec and Trans

UCSR0B |= (1 << RXCIE0);				 //Enable Rec INT

UCSR0C |= (1 << UCSZ01) | (1 << UCSZ00); //Set frame 8-bit, 1 STP

}

void ADC_Init()

{

DDRC	= 0;			//Set PORTC as input for adc

DIDR0	= 0x01;			//Disable Digi input on ADC0

ADMUX = 0;				//Sets Mux selection bits to 0 ADC0 used

ADMUX |= (1 << REFS0);	//Use Vcc Ref voltage selectin 01

ADMUX |= (1 << ADLAR);	//Left adjust ADC Reg, ADCH 8-bit Resolution

ADCSRA |= (1 << ADEN);	//Enable ADC

ADCSRA |= (1 << ADATE); //Set ADC Auto Trig

ADCSRA |= (1 << ADIE);	//Enable Interrupts

ADCSRA |= (1 << ADPS2) | (1 << ADPS1) | (0 << ADPS0); //8MHz, Pre-Scale 64 = 125KHz

ADCSRA |= (1 << ADSC);	//Start Conversion

ADCSRB = 0;			//Free running mode

sei();				//Enable interrupts

}

void outputChr(unsigned char c)

{

UDR0 = c;	//Display Char on Serial

_delay_ms(800);

}

void outputStr(char *c)

{

unsigned int i = 0;	//loop control

while(c[i] != 0)

outputChr(c[i++]);

}

void readTemp()

{

char seeTemp[8];

float	lm34_0;		//For ASCII Temp output

float	lm34_1;		//For showing valued of ADC

while((ADCSRA & (1 << ADIF)) == 0);			//Wait for conversion to finish

/*Conversion to °F*/

lm34_0 = (adcValue * 5.0 / 0x100) * 100.0;	//(ADC * 5 = 200 /256) * 100

dtostrf(lm34_0, 5, 2, seeTemp);				//Float to char conversion

AT_Tx(seeTemp);

}

void AT_Tx(char *t)

{

/*Build Strings for AT+ commands*/

unsigned char CIPStart[]	= "AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\r\n";

unsigned char CIPSend[]		= "AT+CIPSEND=51\r\n";

unsigned char Data[]		= "GET /update?api_key=X5GNOJ6AFIEO0XRP&field1=";

unsigned char temp0			= "t";

unsigned char temp1			= "\r\n";

unsigned char CIPClose[]	= "AT+CIPCLOSE\r\n";

_delay_ms(2000);

outputStr(CIPStart);	//Send Start String

_delay_ms(2000);

outputStr(CIPSend);		//Number of Char being sent

_delay_ms(2000);

outputStr(Data);		//Get command sent

outputStr(t);			//Temperature string added to end of Get command

outputStr("\n\r");		//Enter key

- Schematic

- Flowchart

Timer1

0VF

Read temperature

- THINGSPEAK ACCOUNT AND CHANNEL KEY

- GITHUB LINK

https://github.com/Vasty1995/CPE301

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Yannick Kengne Tatcha