Yannick Kengne Tatcha

CPE301 – SPRING 2018

# Midterm 2

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 1. | COMPONENTS LIST |  |  |
| 2. | TASK 1 & 2 COMPLETE CODE W/ COMMENTS AND SCREENSHOTS |  |  |
| 3. | SCHEMATICS AND SCREENSHOTS |  |  |
| 4. | GitHub Link |  |  |

- COMPONENTS LIST

Needed:

FTDI232R chip

ATMEGA328P Microcontroller

LM34 Sensor

ESP8266-01

- TASK 1 & 2 COMPLETE CODE W/ COMMENTS AND SCREENSHOTS

Transmit Code w/ comments:

#define F_CPU 16000000UL //16 Mhz.

#define BAUD 9600	//Set Baud rate 9600.

#define MYUBRR F_CPU/16/BAUD-1 //Configuration for MYUBRR.

#include <avr/io.h>		//include necessary libraries/files

#include <avr/interrupt.h>

#include <stdbool.h>

#include <string.h>

#include <util/delay.h>

#include "nrf24l01.h"

#include "nrf24l01-mnemonics.h"

#include <stdio.h>

void read_adc(void);	//function to read ADC value

void adc_init(void);	//function to initialize the ADC

void USART_init(unsigned int ubrr);	//Function to initialize USART

void USART_tx_string(char *data);	//Function to print the ADC value

void setup_timer(void);		//function to set up a timer interrupt every 1 second

nRF24L01 *setup_rf(void);	//function to set up nRF24L01 chip

volatile bool rf_interrupt = false;

volatile bool send_message = false;

volatile unsigned int adc_temp;		//variable to hold ADC value read from LM34

char outs[20];				//variable to hold ADC value

void USART_init(unsigned int ubrr)

{

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1<<TXEN0);	//Enable receiver, transmitter & RX interrupt.

UCSR0C = (3<<UCSZ00);  //Asynchronous 8 N 1

}

void USART_Transmit(unsigned char data)

{

while(!(UCSR0A & (1<<UDRE0)));

UDR0=data;				//set UDR0 = data

}

int main(void)

{

adc_init();		//initialize ADC

setup_timer();	//set up a timer1 overflow. Interrupts every 1 second

USART_init(MYUBRR);	//initialize USART

USART_tx_string("Connected!\r\n");  //lets us know if we are connected to the visualizer

_delay_ms(125);		//wait for 125 us

uint8_t to_address[5] = { 0x21, 0x26, 0x32, 0xFF, 0x3E }; //Can pick any combination of hex values so long as it is the same in the receive address.

bool on = false;

nRF24L01 *rf = setup_rf();

sei();		//set enable interrupt

while (true) {

if (rf_interrupt) {

rf_interrupt = false;

int success = nRF24L01_transmit_success(rf);

if (success != 0)

nRF24L01_flush_transmit_message(rf);

}

if (send_message) {

send_message = false;

on = !on;

nRF24L01Message msg;

if (on) memcpy(msg.data, "ON", 3);

else memcpy(msg.data, "OFF", 4);

msg.length = strlen((char *)msg.data) + 1;

nRF24L01_transmit(rf, to_address, &msg);

}

}

return 0;

}

nRF24L01 *setup_rf(void)

{

nRF24L01 *rf = nRF24L01_init();

rf->ss.port = &PORTB;

rf->ss.pin = PB2		//set ss to PB2

rf->ce.port = &PORTB;

rf->ce.pin = PB1;		//set ce to PB1

rf->sck.port = &PORTB;

rf->sck.pin = PB5;		//set sck to PB5

rf->mosi.port = &PORTB;

rf->mosi.pin = PB3;		//set MOSI to PB3

rf->miso.port = &PORTB;

rf->miso.pin = PB4;		//set MISO to PB4

// interrupt on falling edge of INT0 (PD2)

EICRA |= _BV(ISC01);

EIMSK |= _BV(INT0);

nRF24L01_begin(rf);

return rf;

}

// setup timer to trigger interrupt every second when at 1MHz

void setup_timer(void)

{

TIMSK1 |= (1<<TOIE1);			//enable Timer1 overflow interrupt

TCCR1B |= (1<<CS12) | (1<<CS10);	//set timer prescale to 1024

TCNT1 = 49911;		//set TCNT value. Calculated using 65535 - (16MHz/1024-1)

}

// each one second interrupt

ISR(TIMER1_COMPA_vect)

{

send_message = true;		//set send_message = true

}

// nRF24L01 interrupt

ISR(INT0_vect)

{

rf_interrupt = true;		//set rf_interrupt = true

}

void read_adc(void)		//this function is responsible for reading the ADC pins

{

unsigned char i=4;	//declare how many times we will measure temperature per average

adc_temp=0;			//initialize adc_temp to 0

while(i--)			//while i != 0, keep looping

{

ADCSRA |= (1<<ADSC);		//start the first conversion

while(ADCSRA & (1<<ADSC));	//while ADCSRA and 1 is written to ADSC, keep looping

adc_temp+=ADC;			//sum the ADC values

_delay_ms(50);			//wait for 50 ms

}

adc_temp = adc_temp /8;		//take the average of the measured temperatures

}

void adc_init(void)		//this function is responsible for setting up and enabling the ADC

{

ADMUX = (0 << REFS1) | (1<<REFS0) | (0<<ADLAR) | (1<<MUX1) | (1<<MUX0);

ADCSRA = (1<<ADEN) | (0<<ADSC) | (0 << ADATE) | (0<<ADIF) | (0<<ADIE) | (1<<ADPS2)| (0<<ADPS1) | (1<<ADPS0);

}

ISR(TIMER1_OVF_vect)//

{

read_adc();

snprintf(outs,sizeof(outs),"%3d\r\n",adc_temp);	//Change to "%3d F\r\n" for degree output.

USART_tx_string(outs);

_delay_ms(125);

TCNT1 = 49911;	//Reset TCNT1.

}

void USART_tx_string(char *data)

{

while(*data != '\0')

{

while(!(UCSR0A & (1<<UDRE0)));	//print out the variable data

UDR0 = *data;

data++;

}

}

Receive Code w/ comments:

#define F_CPU 16000000UL	//16 MHz

#define BAUD 9600	//Set Baud rate 9600.

#define MYUBRR F_CPU/16/BAUD-1 //Configuration for MYUBRR.

#include <avr/io.h>		//include necessary libraries/files

#include <avr/interrupt.h>

#include <stdbool.h>

#include <string.h>

#include <util/delay.h>

#include <stdio.h>

#include "nrf24l01.h"

#include  "nrf24l01-mnemonics.h"

nRF24L01 *setup_rf(void);

void process_message(char *message);

inline void prepare_led_pin(void);

inline void set_led_high(void);

inline void set_led_low(void);

volatile bool rf_interrupt = false;

void USART_init(unsigned int ubrr);	//Function to initialize USART

void USART_Transmit(unsigned char data); //Function to print the value in Visualizer

int main(void) {

uint8_t address[5] = { 0x21, 0x26, 0x32, 0xFF, 0x3E };//Can pick any combination of hex values so long as it is the same in the receive address.

prepare_led_pin();

sei();		//set enable interrupt

nRF24L01 *rf = setup_rf();

nRF24L01_listen(rf, 0, address);

uint8_t addr[5];

nRF24L01_read_register(rf, CONFIG, addr, 1);

USART_init(MYUBRR);	//initialize USART

while (true) {

if (rf_interrupt) {

rf_interrupt = false;

while (nRF24L01_data_received(rf)) {

nRF24L01Message msg;

USART_tx_string(msg);

nRF24L01_read_received_data(rf, &msg);

process_message((char *)msg.data);

}

nRF24L01_listen(rf, 0, address);

}

}

return 0;

}

nRF24L01 *setup_rf(void) {

nRF24L01 *rf = nRF24L01_init();

rf->ss.port = &PORTB;

rf->ss.pin = PB2;		//set ss to PB2

rf->ce.port = &PORTB;

rf->ce.pin = PB1;		//set ce to PB1

rf->sck.port = &PORTB;

rf->sck.pin = PB5;		//set sck to PB5

rf->mosi.port = &PORTB;

rf->mosi.pin = PB3;		//set MOSI to PB3

rf->miso.port = &PORTB;

rf->miso.pin = PB4;		//set MISO to PB4

// interrupt on falling edge of INT0 (PD2)

EICRA |= _BV(ISC01);

EIMSK |= _BV(INT0);

nRF24L01_begin(rf);

return rf;

}

void process_message(char *message) {

if (strcmp(message, "ON") == 0)

set_led_high();

else if (strcmp(message, "OFF") == 0)

set_led_low();

}

inline void prepare_led_pin(void) {

DDRB |= _BV(PB0);

PORTB &= ~_BV(PB0);

}

inline void set_led_high(void) {

PORTB |= _BV(PB0);

}

inline void set_led_low(void) {

PORTB &= ~_BV(PB0);

}

// nRF24L01 interrupt

ISR(INT0_vect) {

rf_interrupt = true;	//set rf_interrupt = true on INT0 interrupt

}

void USART_init(unsigned int ubrr)

{

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1<<TXEN0);	//Enable receiver, transmitter & RX interrupt.

UCSR0C = (3<<UCSZ00);  //Asynchronous 8 N 1

}

void USART_tx_string(char *str)

{

unsigned int i=0;

while(str[i]!='\0')

{

USART_Transmit(str[i]);		//print out variable str

i++;

}

}

void USART_Transmit(unsigned char data)

{

//check if buffer is empty so that data can be written to transmit (check code example in datasheet)

while(!(UCSR0A & (1<<UDRE0)));

//copy data to be sent to UDR0

UDR0 = data;

}

- SCHEMATICS AND SCREENSHOTS

Block and Breadboard Schematic:

Physical Implementation of Transmitter and Receiver Xmini’s on Breadboard:

Transmitter:

Receiver:

Operation Screenshots:

- GITHUB LINK

https://github.com/Vasty1995

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Yannick Kengne Tatcha